| [COMPANY NAME] [Dept] | [ LOGO ] | TWI — JOB BREAKDOWN SHEET Cover Sub-Assembly — Riveting Op: A-01 ¦ Seq: 3 of 6 ¦ SOP: SOP-A01-001 ¦ WI: WI-A01-001 | DOC NO. TWI-A01-001 | REV Rev. 00 | Trainer: _______________________ Trainee: _______________________ |
| --- | --- | --- | --- | --- | --- |

| INPUT Urea Cover Black (SKU-1234) Rivet #4 Gold x4 (SKU-5678) / WO confirmed | OUTPUT Riveted cover sub-assembly Passed flush inspection / Green tray | LINKED DRAWING DWG-A01-003 Rev. B Cover assy — rivet positions 1→2→3→4 | LINKED SPECS Rivet flush: 0.0 mm ±0.2 Tip gap: 0.3 mm (0.1–0.5) Force: 450N ±20 | PPE: Safety glasses. Cut-resistant gloves (riveting). Steel-toe boots. HAZARD: Pinch point — machine closes at 450N. Sharp tips. Flying rivet debris. |
| --- | --- | --- | --- | --- |

| 1 Retrieve + inspect → seq 1. [ PHOTO / ENGINEERING DRAWING ] Upload annotated photo or drawing Add callout arrows to highlight critical features. KEY POINT: Part number matches WO. Both nubs intact and equal height. No cracks or warping. SPEC: Tip gap: feeler gauge 0.3 mm (accept 0.1–0.5 mm) ✓ Label matches. Nubs identical. Gap in range. No defects. ✗ Chipped nub or gap OOR — reject to red tray — do not proceed. | 2 Seat in fixture → seq 2. [ PHOTO / ENGINEERING DRAWING ] Upload annotated photo or drawing Add callout arrows to highlight critical features. KEY POINT: Lower onto locating pins. Both nubs click into seat. No rocking when center is pressed. SPEC: Cover base flush to fixture surface — zero gap ✓ Click on both tips. Cover flat. No movement when pressed. ✗ Rocks or no click — re-seat carefully — call supervisor if persists. | 3 Load riveter + confirm → seq 3. [ PHOTO / ENGINEERING DRAWING ] Upload annotated photo or drawing Add callout arrows to highlight critical features. KEY POINT: Die label matches WI-A01-001. Force display 450N ±20. Fixture locked. Hands on handles before cycle. SPEC: Machine force: 450N ±20N — read display before every cycle ✓ Die confirmed. Force 430–470N. Fixture locked. Hands clear of closing area. ✗ Force OOR — do NOT cycle. Wrong die = reject batch. Call supervisor. | 4 Rivet 1→2→3→4 + verify each → seq 4. [ PHOTO / ENGINEERING DRAWING ] Upload annotated photo or drawing Add callout arrows to highlight critical features. KEY POINT: Apply rivets in order 1→2→3→4. Depth gauge on each head after cycle — 0.0 mm ±0.2 before moving to next. SPEC: Rivet flush: 0.0 mm ±0.2 mm — depth gauge all 4 positions ✓ 4 rivets in sequence. Each ±0.2 mm. Log signed. Assembly in green tray. ✗ Any rivet OOR — STOP — tag NC — call supervisor — do not continue. |
| --- | --- | --- | --- |

| Process owner: ___________________________ | Approved by: ____________________________ | Trainer: ______________________ Date: __________ | Trainee: ______________________ Date: __________ |
| --- | --- | --- | --- |