| [COMPANY NAME] [Department / Division] | [ COMPANY LOGO ] Replace with logo |
| --- | --- |

| WORK INSTRUCTION Cover Sub-Assembly — Riveting Operation | DOC NO. WI-A01-001 | REVISION Rev. 00 | EFF. DATE [DD-MMM-YYYY] | REVIEW DUE [DD-MMM-YYYY] |
| --- | --- | --- | --- | --- |
| Op ID: A-01 ¦ Seq: 3 of 6 ¦ Ref SOP: SOP-A01-001 ¦ Ref TWI: TWI-A01-001 | PROCESS OWNER [Name / Position] | APPROVER [Name / Position] | QA SIGN-OFF [Name / N/A] |  |

| 1 | SCOPE — INPUT, OUTPUT, EQUIPMENT AND REFERENCES |
| --- | --- |

| INPUT Urea Cover Black (SKU-1234) Rivet #4 Gold x4 (SKU-5678) Work order confirmed | OUTPUT Riveted cover sub-assembly Passed flush inspection In approved (green) tray | EQUIPMENT / STATION Riveter Machine (EQ-003) Assembly fixture (EQ-003-F) Calibration due: [DD-MMM-YYYY] | RELATED DOCUMENTS SOP: Cover Sub-Assembly — Standard Operating Procedure SOP-A01-001 Rev. 00 TWI: Cover Sub-Assembly — Job Breakdown Sheet TWI-A01-001 Rev. 00 Checklist: Cover Sub-Assembly — Quality Inspection Checklist CHK-A01-001 Rev. 00 Data Form: Cover Sub-Assembly — Production Recording Form DAT-A01-001 Rev. 00 Eng. Drawing: Cover Assembly Detail — Rivet Positions 1-2-3-4 DWG-A01-003 Rev. B Spec Sheet: Rivet Flush and Gap Specifications SPEC-A01-001 Rev. 00 |
| --- | --- | --- | --- |

| 2 | OPERATIONAL REQUIREMENTS (5M) |
| --- | --- |

| Man | Qualified assembler — Level 2 certification required. Verify training record before operation. | Machine | Riveter Machine EQ-003. Calibration due: see header. Setup per this WI before each shift. |
| --- | --- | --- | --- |
| Material | Urea Cover Black SKU-1234 — inspect nubs before use. Rivet #4 Gold SKU-5678 — verify lot. | Method | This WI Rev. 00. No active deviations. Follow step sequence exactly — no substitutions. |
| Measure | Depth gauge (GAU-012) — calibration current. Feeler gauge set (GAU-008) — calibration current. | PPE / Safety | Safety glasses mandatory. Cut-resistant gloves during riveting. Steel-toe boots. |

| 3 | SETUP REQUIREMENTS — VERIFY BEFORE STARTING |
| --- | --- |

| PARAMETER | REQUIRED VALUE | TOLERANCE | VERIFICATION METHOD |
| --- | --- | --- | --- |
| Die installed | Die #A01-D4 per station poster | Match exactly | Visual — check die label against setup sheet |
| Machine force | 450 N | ±20 N | Read force display before every cycle |
| Fixture condition | Locating pins intact, no debris | Zero damage | Visual inspection at shift start |
| Rivet supply | Rivet #4 Gold SKU-5678 | SKU match only | Check bin label against work order |

| 4 | PROCESS STEPS |
| --- | --- |

| STEP | DESCRIPTION | CRITICAL PARAM | QC GATE | INSTRUCTION PHOTO |
| --- | --- | --- | --- | --- |
| 1 | Retrieve Urea Cover Black (SKU-1234) from incoming area. Verify part number against work order before moving to station. Material: Urea Cover Black (SKU-1234) | — | ✓ | [ PHOTO / DRAWING — Step 1 ] Upload instruction photo or engineering drawing. |
| 2 | Inspect both alignment nubs — must be intact and equal height. Measure tip gap with feeler gauge. | Gap: 0.3 mm (0.1–0.5) | ✓ | [ PHOTO / DRAWING — Step 2 ] Upload instruction photo or engineering drawing. |
| 3 | Lower cover onto assembly fixture locating pins. Both nubs must click into seat. Press center — no rocking permitted. Material: Assembly fixture (EQ-003-F) | Zero rocking | — | [ PHOTO / DRAWING — Step 3 ] Upload instruction photo or engineering drawing. |
| 4 | Confirm die label matches WI-A01-001. Read force display — must show 450N ±20 before any cycle. Place hands on handles. Material: Riveter Machine (EQ-003) | Force: 450N ±20 | ✓ | [ PHOTO / DRAWING — Step 4 ] Upload instruction photo or engineering drawing. |
| 5 | Apply 4 rivets in sequence: positions 1→2→3→4 per station diagram. After each rivet verify flush with depth gauge before proceeding to next position. Material: Rivet #4 Gold (SKU-5678) — Qty: 4 | Flush: 0.0 mm ±0.2 | ✓ | [ PHOTO / DRAWING — Step 5 ] Upload instruction photo or engineering drawing. |
| 6 | Remove assembly from fixture by lifting straight up — do not twist. Final visual inspection: all 4 rivets flush, cover centered, no fixture marks. Place in green approved tray. Sign quality log. | — | ✓ | [ PHOTO / DRAWING — Step 6 ] Upload instruction photo or engineering drawing. |

| 5 | QUALITY INSPECTION CRITERIA |
| --- | --- |

| CHARACTERISTIC | NOMINAL | USL | LSL | UNIT | GAUGE / METHOD | ACCEPT |
| --- | --- | --- | --- | --- | --- | --- |
| Rivet flush height | 0.0 | +0.2 | -0.2 | mm | Depth gauge GAU-012 — post rivet, all 4 positions | Pass / Fail |
| Cover tip gap | 0.3 | 0.5 | 0.1 | mm | Feeler gauge GAU-008 — before fixture seating | Pass / Fail |
| Machine force | 450 | 470 | 430 | N | Machine force display — before every cycle | Pass / Fail |
| Rivet count | 4 | 4 | 4 | ea | Visual count — all 4 positions confirmed | Pass / Fail |

Current revision only. Previous revisions retained in document control system. Valid only when carrying approved status in ManualCore. Process owner: [Name] Approved by: [Name] QA: [Name / N/A] Date: [DD-MMM-YYYY]

DRAFT — PENDING APPROVAL — NOT FOR PRODUCTION USE